# Multiphase Reaction Engineering (MuRE -- CH3061)

# Assignment 3: Fluidized bed reactor

# Deadline for handing in: 03-03-2026, 23:30

Fluidized bed reactors are commonly used for the processing of powders or catalytic reactions. They have excellent heat and mass transfer properties, but can be complicated to model and control. Due to the fluidized state of the solid particles the entire particle surface is available for reactions and because of the high voidage in the bed FBRs have relatively low pressure drops.

In this assignment we consider the dimerization reaction of ethylene to form 1-butene, using a nickel catalyst:

2C2H4  H2C=CH-CH2-CH3

For reactants and products, the properties of air can be assumed as a simplification. Assume for the first part of the assignment that changes in volume as a result of reaction can be neglected.

Before modelling the reaction the fluidization characteristics of the catalyst need to be looked into.

- Consider a single, spherical, particle in a gas flow (assume properties of air at ambient conditions). Set up a force balance and calculate the required gas velocity to lift the particle. For the particle properties use: ρ = 2850 kg/m3, dp = 60 µm

Hint: assume Stokes law can be used

- When calculating the drag force in a bed of particles the following formula is often used:

FD=3πµdpuε-3.8

Explain the need for the inclusion of ɛ (bed voidage) in this formula and explain qualitatively why the exponent is negative.

- In the lecture we discussed the semi-empirical Ergun equation. When considering small particles, like we do here, this equation can be simplified to:

150(1-ɛmf)ɛmf3Φs2Remf=Ar

Use this to calculate the minimum fluidization velocity (use ɛmf=0.61 and Φs=0.83) and discuss the difference with what you found in question a.

In case you are unfamiliar with the Archimedes number, this is the ratio between gravitational and viscous forces.

Experiments have shown that, for most powders, the value of ɛ of the bed changes very little when increasing the superficial gas velocity beyond the minimum fluidization velocity. The additional gas barely separates the particles further and forms bubbles instead. This means the bed can be modelled as two separate phases, the emulsion phase, which has the fluidized particles in it, and the bubble phase.

- Assuming that for the emulsion phase ɛe = ɛmf when u>umf, derive an expression for the bed height as a function of gas velocity when  u>umf. Assume the bed height at umf, Hmf, and the bubble rise velocity, ub are known.

- Derive an expression for the bubble fraction, ɛb, in the bed.

For the following questions you will use the derived equations from c, d and e to model the fluidized bed reactor. In case you did not manage to derive any of the earlier equations use: umf =0.009m/s, Hb = 0.8m, ɛb = 0.22. Note that these are NOT the correct answers. Please indicate in your report when you used one or more of these values.

- When a two-phase system is assumed, containing bubbles and an emulsion phase, it can be represented as shown in fig[1], where mass transfer takes place between the two phases.

Fig[1]

Set up a balance for how the concentration of ethylene changes in the bubble and emulsion phases over the bed height. For the reaction in the emulsion phase use:

rA= -γeKrCA2

Where γe  is the fraction of solids in the emulsion phase. Assume no solids to be present in the bubble phase. For mass transfer use constant ‘K’ (unit 1/s).

- Write the equations you derived in f as one single differential equation to express how the concentration of ethylene in the bubble changes over reactor height.

- Fill in all the missing parts in the provided Python code. Use this to solve the differential equation and calculate the final conversion in this reactor. Plot the concentrations of ethylene in both the bubble and emulsion phase.

- For this assignment we assumed that no changes in volume occur. This is a valid assumption only if the volume change is very small. Use the found conversion for ethylene to calculate the volume change. Note that the gas entering the reactor is not pure ethylene (see python code). Conclude, based on your answer, whether the assumption of no volume change was valid. Assume ambient conditions.

- Finally, propose a way to reach higher conversion in this reactor. When you choose to change a parameter, don’t just say: change ‘x’, but explain how this parameter can be changed and why it influences the conversion (explain the underlying mechanism). Calculate the new conversion based on your proposed optimization.