# Multiphase Reactor Engineering (MuRE -- CH3061)

# Assignment 5: Bubble Column Reactors

# Deadline for handing in: 17-03-2024, 23:30

In this assignment, we will focus on the chemical absorption of CO2 in NaOH solution, a common process in scrubbers to absorb CO2. Initially, the gaseous CO2 is absorbed in the liquid phase:

CO2(g) →CO2(aq)									(1)

The sodium hydroxide dissociates in aqueous solutions into sodium and hydroxide ions:

NaOH ⇌Na(aq)++OH(aq)-								(2)

Carbon dioxide reacts with the hydroxide ions to carbonate and bicarbonate ions according to the following equilibrium reactions:

CO2(aq)+ OH- ⇌HCO3-								(3)

HCO3-+ OH-⇌CO32-+H2O							(4)

For simplification, neglect sodium ions as they do not participate in the reaction. Additionally, ignore equation (4) assuming that only (HCO3-) forms. Furthermore, for equation (3), consider only the forward reaction. The process is carried out in a bubble column: the liquid phase is in batch and the gas phase (which is 30 mol% CO2 and 70 mol% N2) is the continuous phase with the flowrate of 70.0 l/min. Bubbles rise by buoyancy. The column diameter is 1.5 m, and the height of the column is 3.0 m. The initial concentration of NaOH is 0.04 mol/l. Surface tension is 0.073 N/m. Bubble diameter is db = 10.0 mm. Process condition is at atmospheric pressure and 298.15 K.

- Calculate the bubble terminal velocity. (you can use what you learned in fluidized beds, instead of solid particles you have “fluid particles” here) - Use this equation for calculating the drag coefficient: CD= 23 Eo , Eo is Eotvos number.

- Determine Henry’s constant.

- Consider the liquid in the bubble column as ideally mixed and write down the molar balance equation for CO2 and  OH- in the aqueous phase (ignore the bubble volume reduction during the process). In case you did not manage to calculate the Henry’s constant 5×10-1.

- Calculate kLa. (kLa =0.467* ug0.82 )

- Calculate the gas hold up. What is the flow regime and discuss the effects of the flow regime and how we can change it, if needed.

- Plot the concentration for CO2 and OH- as a function of time over the first 10 seconds after introducing bubbles into the bubble column. Evaluate ways for improvement.